# K900SDK iOS Integration Guide

English edition

Documentation baseline: 2026-09-09

Maintained source: XingYi developer center / SDK / iOS / K900SDK. This document is generated from the same shared data.

Review scope: SDK source and protocol reference material. Source behavior and historical protocol information are distinguished. Release binary compilation and physical-device firmware validation have not been completed.

Unconfirmed reply codes, fields, completion criteria and examples are intentionally blank. Blank does not mean no reply. Update the developer center first, then regenerate both manuals.

## Contents

1  Overview and version

2  Project integration and permissions

3  Quick start

4  BLE integration flow

5  AI photo capture and image transfer

6  Wi-Fi file transfer

7  LC3 real-time audio transfer

8  OTA update flow

9  API reference

10  Reply status and error handling

11  Using the demo

12  Updates and migration

# 1  Overview and version

K900SDK connects iOS applications to K900 glasses. The actual module name is XingYiGlassesSDK and the resource bundle is XYSDK.bundle.

Use sendAiph to request AI photo capture and image transfer. Device control, queries, photos, video and audio commands are documented in the API reference.

The developer center is the maintained source of integration information. Chinese and English manuals share API declarations, command codes, parameter tables and reply data. Unknown replies are left blank; a blank does not mean no reply exists.

# 2  Project integration and permissions

Add XingYiGlassesSDK.framework and XYSDK.bundle to the app target. Link the framework and include the bundle in Copy Bundle Resources. The current source target builds a static framework; choose Do Not Embed for that artifact.

The current SDK target is configured for iOS 15.6. Confirm minimum OS, Xcode and simulator architecture support against the release artifact. Use a physical iPhone for BLE integration.

Set NSBluetoothAlwaysUsageDescription for BLE. Configure NSLocalNetworkUsageDescription for file transfer and OTA, and enable Hotspot Configuration when using the system hotspot API. Request microphone and photo-library permissions only for features that actually use those resources.

Demo UI libraries, cloud AI services and third-party credentials are not mandatory core BLE dependencies for every app.

#import <XingYiGlassesSDK/XingYiGlassesSDK.h>

NSString *version = [[XYBleManager sharedInstance] xy_sdkVersion];

# 3  Quick start

After importing the SDK and configuring permissions, register callbacks before scanning. Retain the discovered CBPeripheral, stop scanning after device selection and connect. Once communication is ready, query battery level and handle sr_batv with status string 0.

A connection callback does not imply communication readiness. Manage singleton callbacks through one session owner, update UI on the main thread and stop sending business commands after disconnection until reconnected.

XYBleManager *ble = [XYBleManager sharedInstance];

ble.receiveDataBlock = ^(XYBleRespondModel *response) {

if ([response.cmd isEqualToString:@"sr_batv"] &&

[response.status isEqualToString:@"0"]) {

NSLog(@"Battery: %@", response.data[@"pt"]);

}

};

// Call after communication readiness is confirmed.

[ble sendGetBatteryVol];

# 4  BLE integration flow

Register discovery, connection and data callbacks before scanning. Retain the CBPeripheral returned by discovery; stop scanning and connect after device selection. Manage singleton callbacks in one session owner because a new assignment replaces the previous callback.

connectStatusBlock(YES) currently occurs before characteristic discovery and notification setup complete. Peripheral connection, BLE communication readiness and device system readiness are separate states. Do not assume all commands are ready in that callback.

xy_startScanBleDevice currently cancels existing connections, so do not restart scanning accidentally during a session. openHeart sends one heartbeat and does not create a repeating timer. Dispatch UI updates to the main thread.

XYBleManager *ble = [XYBleManager sharedInstance];

ble.receiveDataBlock = ^(XYBleRespondModel *response) {

if ([response.cmd isEqualToString:@"sr_batv"] &&

[response.status isEqualToString:@"0"]) {

NSLog(@"Battery: %@", response.data[@"pt"]);

}

};

// Call after communication readiness is confirmed.

[ble sendGetBatteryVol];

# 5  AI photo capture and image transfer

Call sendAiph after BLE communication and the target device feature are ready. The method takes no parameters and sends C=cs_aiph, an empty string B and W=1.

sendAiph is distinct from normal photo capture with sendTakePhoto: and parameterized BLE capture with sendTakeBlePhoto:width:height:quality:. Do not reuse a reply contract or image dimensions from one method for another.

The SDK provides a general fileReceiveDataBlock callback. Current material does not establish a command-specific binding between AI capture and that callback, an image format or a completion reply. The command-specific reply code, fields and completion criteria are left blank for the target firmware.

Copy received file data before asynchronous storage. A void method return does not mean image transfer has completed.

[[XYBleManager sharedInstance] sendAiph];

// Exact request sent by the SDK:

// {"C":"cs_aiph","B":"","W":1}

# 6  Wi-Fi file transfer

Register BLE and socket callbacks, request a hotspot with openType=0, read hotspot information from sr_apsr, join it and connect the socket before requesting files. The implementation currently uses 192.168.43.1 on port 6001.

XYSocketManager callback data is a cumulative buffer. The SDK clears it after the completion callback returns, so copy it before asynchronous use. Acknowledge receipt only after successful storage.

sendFileRecv:fileName: currently omits JSON string quotes around fname. A normal disconnect and a successful connection may both report a nil error. Resolve or clarify these behaviors before production integration; documentation does not imply they are fixed.

Protocol file types are 49 photo, 50 video, 51 audio, 52 BLE photo and 255 all. Do not use the all-files type for a single-file acknowledgment.

# 7  LC3 real-time audio transfer

sendSound: currently maps Start=1, Chat=6, Stop=5 and No_Sound=2. The legacy description reverses Chat and Stop. These are source-confirmed transmitted values; verify device semantics separately.

sendStartLC3 and sendStopLC3 send enable_custom_audio_tx with enable=1 and enable=0 respectively. Audio is provided through lc3ReceiveDataBlock. Sample rate, frame length, channels and decoder parameters are not confirmed by the current material.

# 8  OTA update flow

Query device versions and select a compatible update package. A differential system package must match the current source version. Open an update hotspot with openType=1 and use XYOTAManager to send the data.

The current APK=0, MTK=1 and BES=2 enums map to transmitted protocol types 54, 55 and 56. Verify device support for each type against firmware.

A successful completionBlock means socket packet writes have completed, not that installation has finished. Wait for device update events and verify the target version after reboot and reconnection. Do not present 100% transfer as update success.

# 9  API reference

C is the command code, B is payload data and S is reply status. S=0 means success; unsolicited events may omit S. Missing S must not be treated as success. XYBleRespondModel.status is a string; compare it exactly.

Preserve the JSON-string or object encoding actually used by the SDK for B. The complete meaning of W is unconfirmed; cs_aiph uses W=1. Examples come from source or historical protocol material, not verified device captures. A void return does not indicate device completion.

| No. | Request / event | Feature | Reply code |
| --- | --- | --- | --- |
| 1 | cs_hrt | Heartbeat | sr_hrt |
| 2 | cs_stm | Set time | sr_stm |
| 3 | cs_batv | Read battery | sr_batv |
| 4 | cs_syvr | Read firmware versions | sr_syvr |
| 5 | cs_gtst | Read storage information | sr_gtst |
| 6 | cs_shut | Shut down device | sr_shut |
| 7 | cs_rebt | Restart device | sr_rebt |
| 8 | cs_wku | Wake system |  |
| 9 | cs_slp | Sleep system |  |
| 10 | cs_pho | Take photo | sr_pho |
| 11 | cs_ctph | Continuous photography | sr_ctph |
| 12 | cs_vdo | Start video recording | sr_vdo |
| 13 | cs_ado | Start audio recording | sr_ado |
| 14 | cs_opap | Open glasses hotspot | sr_apsr |
| 15 | cs_csap | Close glasses hotspot | sr_csap |
| 16 | cs_asfl | Request media files | sr_asfl |
| 17 | cs_clst | Clear media files | sr_clst |
| 18 | cs_gmst | Read media settings | sr_gmst |
| 19 | cs_phst | Set photo parameters | sr_phst |
| 20 | cs_vdst | Set video parameters | sr_vdst |
| 21 | cs_adst | Set audio parameters | sr_adst |
| 22 | sr_stat | System status event | sr_stat |
| 23 | cs_ivww | Request AI wake | sr_ivww |
| 24 | cs_wfsta | Send hotspot credentials | sr_wfsta |
| 25 | cs_phoalg | Set image processing | sr_phoalg |
| 26 | cs_vdoeis | Set video stabilization | sr_vdoeis |
| 27 | cs_algeis | Read image and stabilization settings | sr_algeis |
| 28 | cs_swst | Read interaction switches |  |
| 29 | cs_swit | Set interaction switch | sr_swit |
| 30 | cs_aiph | AI photo capture and image transfer |  |
| 31 | cs_wfop | Set Wi-Fi connection |  |
| 32 | cs_sound | Synchronize AI state | sr_sound |
| 33 | cs_stvd | Stop video recording | sr_stvd |
| 34 | cs_stad | Stop audio recording | sr_stad |
| 35 | cs_wrst | Read wear state | sr_wrst |
| 36 | cs_btst | Read BT connection state | sr_btst |
| 37 | cs_blna | Set BLE name | sr_blna |
| 38 | cs_gbtna | Read BT name | sr_gbtna |
| 39 | cs_btaddr | Read MAC address | sr_btaddr |
| 40 | cs_vol | Set volume | sr_vol |
| 41 | cs_ledon | Turn LED on | sr_ledon |
| 42 | cs_ledoff | Turn LED off | sr_ledoff |
| 43 | sr_adota | OTA progress event | sr_adota |
| 44 | cs_fbvol | Set swipe volume control | sr_fbvol |
| 45 | cs_hfpst | Read HFP call state | sr_hfpst |
| 46 | cs_flts | AI file transfer flow control |  |
| 47 | enable_custom_audio_tx | Control LC3 audio transmission |  |
| 48 | sr_vcmd | Voice command event | sr_vcmd |
| 49 | sr_tpevt | Touch event | sr_tpevt |
| 50 | sr_keyevt | Button event | sr_keyevt |
| 51 | sr_stai | AI stop event | sr_stai |
| 52 | sr_log | Device log event | sr_log |
| 53 | cs_sdfl | Send file notification |  |
| 54 | cs_wkcfm | Confirm AI session |  |
| 55 | cs_wsdelay | Set wake prompt delay |  |
| 56 | enable_custom_audio_rx | Control LC3 audio reception |  |
| 57 | enable_hfp_audio_server | Control HFP audio service |  |
| 58 | cs_vdroixy | Set video ROI | sr_vdroixy |
| 59 | sr_ivcw | Voice wake closed event | sr_ivcw |
| 60 | cs_vbit | Set video bitrate | sr_vbit |
| 61 | cs_blecfm | Play connected prompt | sr_blecfm |
| 62 | cs_wbdc | Set BT disconnect on wear-off | sr_wbdc |
| 63 | cs_nwai | Allow AI while not worn | sr_nwai |
| 64 | cs_woft | Set wear-off delay | sr_woft |
| 65 | cs_syrd | Read system readiness |  |
| 66 | cs_flrv | Acknowledge received file |  |
| 67 | cs_btna | Set BT name | sr_btna |
| 68 | cs_svad | Set directional voice pickup |  |
| 69 | cs_wfcon | Connect glasses to Wi-Fi |  |
| 70 | cs_syntm | Synchronize time string |  |
| 71 | cs_bleph | Take BLE photo |  |
| 72 | cs_en_campa | Enable custom camera parameters |  |
| 73 | cs_set_campa | Set custom camera parameters |  |
| 74 | cs_get_campa | Read camera parameters |  |
| 75 | cs_get_camparange | Read camera parameter ranges |  |

## 9.1  cs_hrt  Heartbeat

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)openHeart;

Synchronous heartbeat packet

### Usage

[[XYBleManager sharedInstance] openHeart];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_hrt","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_hrt |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| vt | integer | Voltage in millivolts |
| pt | integer | Battery percentage |
| charg |  | Charging state, values not specified |
| ready |  | System readiness, values not specified |

Reply example

{"C":"sr_hrt","S":0,"B":{"vt":4192,"pt":81,"charg":0,"ready":1}}

## 9.2  cs_stm  Set time

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetTime;

Set time

### Usage

[[XYBleManager sharedInstance] sendSetTime];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| mill | integer | Current Unix timestamp in milliseconds |

B encoding: JSON string; empty string when no data is present

{"C":"cs_stm","B":"{\"mill\":1731920125169}"}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_stm |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

{"C":"sr_stm","S":0,"B":""}

## 9.3  cs_batv  Read battery

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendGetBatteryVol;

Obtain electricity

### Usage

[[XYBleManager sharedInstance] sendGetBatteryVol];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_batv","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_batv |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| vt | integer | Voltage in millivolts |
| pt | integer | Battery percentage |

Reply example

{"C":"sr_batv","S":0,"B":{"vt":4192,"pt":81}}

## 9.4  cs_syvr  Read firmware versions

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendGetSystemVersion;

Get version

### Usage

[[XYBleManager sharedInstance] sendGetSystemVersion];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_syvr","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_syvr |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| app |  | Glasses application version |
| sys | string | Glasses system firmware version |
| dpj | string | Microcontroller version |

Reply example

Example content

## 9.5  cs_gtst  Read storage information

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendGetStorageInfo;

Get stored information

### Usage

[[XYBleManager sharedInstance] sendGetStorageInfo];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_gtst","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_gtst |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| total_size |  | Total storage capacity, unit not specified |
| free_size |  | Free storage capacity, unit not specified |
| phonum | integer | Photo count |
| vdonum | integer | Video count |
| adonum | integer | Audio count |

Reply example

Example content

## 9.6  cs_shut  Shut down device

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)shutDevice;

Device shutdown

### Usage

[[XYBleManager sharedInstance] shutDevice];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_shut","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_shut |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.7  cs_rebt  Restart device

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)rebootDevice;

Device restart

### Usage

[[XYBleManager sharedInstance] rebootDevice];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_rebt","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_rebt |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.8  cs_wku  Wake system

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

No corresponding wrapper in the public header.

### Request parameters

No method payload parameters.

{"C":"cs_wku","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.9  cs_slp  Sleep system

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

No corresponding wrapper in the public header.

### Request parameters

No method payload parameters.

{"C":"cs_slp","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.10  cs_pho  Take photo

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendTakePhoto:(NSString *)pictureName;

Take a picture
@param pictureName Picture name: 20241126162655

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSString * pictureName

[[XYBleManager sharedInstance] sendTakePhoto:pictureName];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| fname | string | Photo name without the .jpg extension |

B encoding: JSON string; empty string when no data is present

{"C":"cs_pho","B":"{\"fname\":\"20260909143000\"}"}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_pho |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.11  cs_ctph  Continuous photography

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendContinuePhoto;

Continuous shooting

### Usage

[[XYBleManager sharedInstance] sendContinuePhoto];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| time | string | Current timestamp generated by the SDK |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_ctph |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.12  cs_vdo  Start video recording

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendVideoRecord:(NSString *)videoName;

Video recording
@param videoName Video name: 20241126162655

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSString * videoName

[[XYBleManager sharedInstance] sendVideoRecord:videoName];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| fname | string | Video name without the .mp4 extension |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_vdo |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.13  cs_ado  Start audio recording

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendAudioRecord:(NSString *)audioName;

Audio recording
@param audioName Audio name: 20241126162655

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSString * audioName

[[XYBleManager sharedInstance] sendAudioRecord:audioName];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| fname | string | Recording name without the .mp3 extension |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_ado |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.14  cs_opap  Open glasses hotspot

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)requestServerOpenHotspotWithOpenType:(NSInteger )openType;

Turn on Wifi hotspot
@param openType 0: normal, 1: OTA upgrade

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSInteger openType

[[XYBleManager sharedInstance] requestServerOpenHotspotWithOpenType:openType];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| platform | integer | 1 for OTA mode, empty B in normal mode |

B encoding: JSON string; empty string when no data is present

{"C":"cs_opap","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_apsr |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| succ | boolean | Whether hotspot creation succeeded |
| ssid | string | Hotspot SSID |
| pwd | string | Hotspot password |

Reply example

Example content

## 9.15  cs_csap  Close glasses hotspot

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)closeHotspot;

Turn off Wifi hotspot

### Usage

[[XYBleManager sharedInstance] closeHotspot];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_csap","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_csap |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.16  cs_asfl  Request media files

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)askFile:(XYFileType )fileType;

Request file transfer
@param fileType File type

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: XYFileType fileType

[[XYBleManager sharedInstance] askFile:fileType];

The SDK uses 51 for audio and 52 for BLE photos. The legacy audio value 0x34 is not retained.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| ftype | integer | 49 photo, 50 video, 51 audio, 52 BLE photo, 255 all |

B encoding: JSON string; empty string when no data is present

{"C":"cs_asfl","B":"{\"ftype\":49}"}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_asfl |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| filecount | integer | Total number of transferred files |

Reply example

Example content

## 9.17  cs_clst  Clear media files

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendClearAllMedia:(XYFileType )fileType;

Request to empty multimedia
@param fileType File type

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: XYFileType fileType

[[XYBleManager sharedInstance] sendClearAllMedia:fileType];

Do not blindly retry deletion. The SDK audio type is 51.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| ftype | integer | 49 photo, 50 video, 51 audio, 52 BLE photo, 255 all |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_clst |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.18  cs_gmst  Read media settings

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendGetMediaSettings;

Get multimedia Settings

### Usage

[[XYBleManager sharedInstance] sendGetMediaSettings];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_gmst","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_gmst |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| pw | integer | Photo width |
| ph | integer | Photo height |
| vw | integer | Video width |
| vh | integer | Video height |
| vq | integer | 0 low, 1 medium, 2 high |
| vs | integer | Video duration in seconds |
| as | integer | Audio duration in seconds |

Reply example

Example content

## 9.19  cs_phst  Set photo parameters

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetPhotoParam:(XYPhotoArgumentType )photoArgumentType;

Setting photo parameters
@param photoArgumentType Photo parameter

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: XYPhotoArgumentType photoArgumentType

[[XYBleManager sharedInstance] sendSetPhotoParam:photoArgumentType];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| w | integer | Photo width selected through XYPhotoArgumentType |
| h | integer | Photo height paired with width |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_phst |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| succ | boolean | Whether configuration succeeded |
| w | integer | Photo width |
| h | integer | Photo height |

Reply example

Example content

## 9.20  cs_vdst  Set video parameters

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetVideoParam:(XYVideoArgumentSizeType )videoArgumentSizeType videoArgumentQualityType:(XYVideoArgumentQualityType )videoArgumentQualityType videoArgumentDurationType:(XYVideoArgumentDurationType )videoArgumentDurationType;

Setting recording parameters
@param videoArgumentSizeType Video size
@param videoArgumentQualityType Video quality
@param videoArgumentDurationType Recording duration

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: XYVideoArgumentSizeType videoArgumentSizeType, XYVideoArgumentQualityType videoArgumentQualityType, XYVideoArgumentDurationType videoArgumentDurationType

[[XYBleManager sharedInstance] sendSetVideoParam:videoArgumentSizeType videoArgumentQualityType:videoArgumentQualityType videoArgumentDurationType:videoArgumentDurationType];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| w | integer | Video width, 1920, 1280 or 640 |
| h | integer | Matching height, 1080, 720 or 480 |
| q | integer | 0 low, 1 medium, 2 high |
| s | integer | Duration, 30, 60, 180 or 300 seconds |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_vdst |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| succ | boolean | Whether configuration succeeded |
| w | integer | Video width |
| h | integer | Video height |
| q | integer | Video quality |
| s | integer | Video duration in seconds |

Reply example

Example content

## 9.21  cs_adst  Set audio parameters

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetAudioParam:(XYAudioArgumentDurationType )audioArgumentType;

Setting recording Parameters
@param audioArgumentType Recording parameter types

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: XYAudioArgumentDurationType audioArgumentType

[[XYBleManager sharedInstance] sendSetAudioParam:audioArgumentType];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| s | integer | Duration, 60, 180, 300, 600, 1800, 3600 or 7200 seconds |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_adst |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| succ | boolean | Whether configuration succeeded |
| s | integer | Audio duration in seconds |

Reply example

Example content

## 9.22  sr_stat  System status event

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

Unsolicited device event; use the shared receive callback.

### Event data

| Item | Value |
| --- | --- |
| Event code | sr_stat |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| type | string | Status type, sys listed by reference |
| state | integer | For sys, 1 means system booting |
| msg | string | Status message |

Event example

Example content

## 9.23  cs_ivww  Request AI wake

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendWakeUp;

Wake up

### Usage

[[XYBleManager sharedInstance] sendWakeUp];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_ivww","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_ivww |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| lang | string | unknow for button-related wake, en English, zh Chinese, see src for source |
| src | integer | 0 voice, 1 single touch, 2 button |

Reply example

Example content

## 9.24  cs_wfsta  Send hotspot credentials

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendWifiSTA:(NSString *)ssid pwd:(NSString *)pwd ip:(NSString *)ip;

Set up mobile phone wifi
@param ssid ssid
@param pwd pwd
@param ip ip

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSString * ssid, NSString * pwd, NSString * ip

[[XYBleManager sharedInstance] sendWifiSTA:ssid pwd:pwd ip:ip];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| ssid | string | Hotspot SSID |
| pwd | string | Hotspot password |
| ip | string | IP address |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_wfsta |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| ip | string | Glasses IP address |

Reply example

Example content

## 9.25  cs_phoalg  Set image processing

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetPhoalg:(BOOL )isOn;

Set the on/off image algorithm
@param isOn Whether to open or not

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: BOOL isOn

[[XYBleManager sharedInstance] sendSetPhoalg:isOn];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| enable | integer | 1 on, 0 off |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_phoalg |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| succ | boolean | Whether configuration succeeded |

Reply example

Example content

## 9.26  cs_vdoeis  Set video stabilization

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetVdoeis:(BOOL )isOn;

Set to enable or disable video image stabilization
@param isOn Whether to open or not

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: BOOL isOn

[[XYBleManager sharedInstance] sendSetVdoeis:isOn];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| enable | integer | 1 on, 0 off |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_vdoeis |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| succ | boolean | Whether configuration succeeded |

Reply example

Example content

## 9.27  cs_algeis  Read image and stabilization settings

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendAlgeis;

Obtain the status of the image algorithm/video anti-shake switch

### Usage

[[XYBleManager sharedInstance] sendAlgeis];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_algeis","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_algeis |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| succ | boolean | Whether the request succeeded |
| phoalgenable |  | Image algorithm state, 1 on 0 off, type not specified |
| vdoeisenable |  | Video stabilization state, 1 on 0 off, type not specified |

Reply example

Example content

## 9.28  cs_swst  Read interaction switches

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSwst;

Obtain the status of the intelligent interactive switch

### Usage

[[XYBleManager sharedInstance] sendSwst];

The legacy reply code conflicts with its example. Reply is left blank.

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_swst","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.29  cs_swit  Set interaction switch

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetSwit:(NSInteger )type isOn:(BOOL )isOn;

Set the intelligent interactive switch
@param type 1. Wearing detection, 2. Photo prompt, 3. Video prompt, 4. Audio prompt, 5. Button wake-up, 6. Touch, 7. Flashing light, 8. Voice wake-up
@param isOn Whether to open or not

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSInteger type, BOOL isOn

[[XYBleManager sharedInstance] sendSetSwit:type isOn:isOn];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| type | integer | 1 wear, 2 photo prompt, 3 video prompt, 4 audio prompt, 5 key wake, 6 touch, 7 LED, 8 voice wake |
| switch | integer | 1 on, 0 off, encoded as an integer by SDK |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_swit |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.30  cs_aiph  AI photo capture and image transfer

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendAiph;

AI photography

### Usage

[[XYBleManager sharedInstance] sendAiph];

Use public method sendAiph. It has no parameters and sends W=1. Command-specific image reply and completion contract are left blank.

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_aiph","B":"","W":1}

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.31  cs_wfop  Set Wi-Fi connection

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendWifiWfop:(NSString *)ssid pwd:(NSString *)pwd isOn:(BOOL )isOn;

Set the wifi switch
@param ssid ssid
@param pwd pwd
@param isOn Whether to open or not

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSString * ssid, NSString * pwd, BOOL isOn

[[XYBleManager sharedInstance] sendWifiWfop:ssid pwd:pwd isOn:isOn];

The legacy reply code conflicts with its example. Reply is left blank.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| ssid | string | Wi-Fi SSID |
| pwd | string | Wi-Fi password |
| enable | integer | 1 on, 0 off |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.32  cs_sound  Synchronize AI state

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSound:(XYAIStatusType )statusType;

Synchronize the AI status
@param statusType Status type

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: XYAIStatusType statusType

[[XYBleManager sharedInstance] sendSound:statusType];

SDK maps Chat to 6 and Stop to 5, opposite to the legacy reference. These are the current transmitted values, firmware semantics require verification.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| type | integer | SDK mapping Start=1, Chat=6, Stop=5, No_Sound=2 |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_sound |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.33  cs_stvd  Stop video recording

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendStopVideoRecord;

Stop video recording

### Usage

[[XYBleManager sharedInstance] sendStopVideoRecord];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_stvd","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_stvd |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.34  cs_stad  Stop audio recording

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendStopAudioRecord;

Stop audio recording

### Usage

[[XYBleManager sharedInstance] sendStopAudioRecord];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_stad","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_stad |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.35  cs_wrst  Read wear state

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendGetWearState;

Get the wearing status

### Usage

[[XYBleManager sharedInstance] sendGetWearState];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_wrst","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_wrst |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| on | integer | 1 worn, 0 not worn |

Reply example

Example content

## 9.36  cs_btst  Read BT connection state

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendGetBTConnectState;

Get the BT Bluetooth connection status

### Usage

[[XYBleManager sharedInstance] sendGetBTConnectState];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_btst","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_btst |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| on | integer | 1 connected, 0 disconnected |

Reply example

Example content

## 9.37  cs_blna  Set BLE name

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetBleName:(NSString *)bleName;

Set the Ble Bluetooth name
@param bleName Ble Bluetooth name

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSString * bleName

[[XYBleManager sharedInstance] sendSetBleName:bleName];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| blename | string | New BLE name |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_blna |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.38  cs_gbtna  Read BT name

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendGetBTName;

Get the BT Bluetooth name

### Usage

[[XYBleManager sharedInstance] sendGetBTName];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_gbtna","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_gbtna |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| btname | string | BT name |

Reply example

Example content

## 9.39  cs_btaddr  Read MAC address

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendGetMacAddress;

Get the mac address

### Usage

[[XYBleManager sharedInstance] sendGetMacAddress];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_btaddr","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_btaddr |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| btaddr | string | MAC address |

Reply example

Example content

## 9.40  cs_vol  Set volume

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetVolume:(NSInteger)volume;

Set volume
@param volume Volume, range 0~15

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSInteger volume

[[XYBleManager sharedInstance] sendSetVolume:volume];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| vol | integer | Volume from 0 to 15 |

B encoding: JSON string; empty string when no data is present

{"C":"cs_vol","B":"{\"vol\":10}"}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_vol |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

{"C":"sr_vol","S":0}

## 9.41  cs_ledon  Turn LED on

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

No corresponding wrapper in the public header.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| led | integer | 0 red, 1 green, 2 blue, 3 orange, 4 white |
| ontime | integer | On duration in milliseconds |
| offtime | integer | Off duration in milliseconds |
| count | integer | Repeat count |

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_ledon |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.42  cs_ledoff  Turn LED off

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

No corresponding wrapper in the public header.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| led | integer | 0 red, 1 green, 2 blue, 3 orange, 4 white |

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_ledoff |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.43  sr_adota  OTA progress event

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

Unsolicited device event; use the shared receive callback.

### Event data

| Item | Value |
| --- | --- |
| Event code | sr_adota |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| type | string | update progress, success completed, error failed |
| progress | integer | 0 to 100 |

Event example

{"C":"sr_adota","B":{"type":"update","progress":35}}

## 9.44  cs_fbvol  Set swipe volume control

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

No corresponding wrapper in the public header.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| switch | integer | 1 on, 0 off |

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_fbvol |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.45  cs_hfpst  Read HFP call state

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

No corresponding wrapper in the public header.

### Request parameters

No method payload parameters.

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_hfpst |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| on | integer | 0 no active call, 1 active call |

Reply example

Example content

## 9.46  cs_flts  AI file transfer flow control

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

No corresponding wrapper in the public header.

This method is internal, not public. The implementation sends state, index and W=1. Automatic retransmission calls are commented out.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| state | integer | 1 requests next packet, 0 signals error or retransmission |
| index | integer | Expected packet index starting at 0 |

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.47  enable_custom_audio_tx  Control LC3 audio transmission

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendStartLC3;

Enable LC3 audio transmission

- (void)sendStopLC3;

Disable LC3 audio transmission

### Usage

[[XYBleManager sharedInstance] sendStartLC3];

[[XYBleManager sharedInstance] sendStopLC3];

The SDK sends TX. The RX title and example in the legacy reference do not describe this method.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| enable | integer | 1 on, 0 off |

B encoding: JSON string; empty string when no data is present

{"C":"enable_custom_audio_tx","B":"{\"enable\":0}"}

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.48  sr_vcmd  Voice command event

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

Unsolicited device event; use the shared receive callback.

### Event data

| Item | Value |
| --- | --- |
| Event code | sr_vcmd |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| cmd | integer | 1 photo, 2 start video, 3 stop video, 4 answer call, 5 reject or hang up, 6 volume up, 7 volume down, 20 timeout, 21 invalid command |

Event example

Example content

## 9.49  sr_tpevt  Touch event

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

Unsolicited device event; use the shared receive callback.

### Event data

| Item | Value |
| --- | --- |
| Event code | sr_tpevt |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| type | integer | 0 tap, 1 double tap, 2 triple tap, 3 long press, 4 forward swipe, 5 backward swipe, 6 up, 7 down |

Event example

Example content

## 9.50  sr_keyevt  Button event

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

Unsolicited device event; use the shared receive callback.

### Event data

| Item | Value |
| --- | --- |
| Event code | sr_keyevt |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| button | integer | 0 left button, 1 right button |
| type | integer | 0 single press, 1 long press, 2 double press, 3 triple press |

Event example

Example content

## 9.51  sr_stai  AI stop event

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

Unsolicited device event; use the shared receive callback.

### Event data

| Item | Value |
| --- | --- |
| Event code | sr_stai |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |
| type | integer | 0 means AI stopped or ended |

Event example

Example content

## 9.52  sr_log  Device log event

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

Unsolicited device event; use the shared receive callback.

The legacy log and body field names conflict. Fields are left blank.

### Event data

| Item | Value |
| --- | --- |
| Event code | sr_log |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Event example

Example content

## 9.53  cs_sdfl  Send file notification

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

No corresponding wrapper in the public header.

### Request parameters

No method payload parameters.

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.54  cs_wkcfm  Confirm AI session

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendAiConfirm;

AI confirm

### Usage

[[XYBleManager sharedInstance] sendAiConfirm];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_wkcfm","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.55  cs_wsdelay  Set wake prompt delay

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSoundDelay:(NSInteger )mills;

Delay playing the prompt tone
@param mills Time

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSInteger mills

[[XYBleManager sharedInstance] sendSoundDelay:mills];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| mills | integer | Wake prompt delay, SDK preserves the field name mills |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.56  enable_custom_audio_rx  Control LC3 audio reception

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

No corresponding wrapper in the public header.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| enable | integer | 1 on, 0 off |

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.57  enable_hfp_audio_server  Control HFP audio service

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

No corresponding wrapper in the public header.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| enable | integer | 1 on, 0 off |

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.58  cs_vdroixy  Set video ROI

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendVideoRoixyWithX:(NSInteger)x y:(NSInteger)y;

Set video ROI coordinates
@param x X coordinate, Range 0~1144
@param y Y coordinate, Range 0~1020

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSInteger x, NSInteger y

[[XYBleManager sharedInstance] sendVideoRoixyWithX:x y:y];

The SDK sends x and y only, not w and h listed in the legacy reference.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| x | integer | Header range 0 to 1144 |
| y | integer | Header range 0 to 1020 |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_vdroixy |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.59  sr_ivcw  Voice wake closed event

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

Unsolicited device event; use the shared receive callback.

### Event data

| Item | Value |
| --- | --- |
| Event code | sr_ivcw |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Event example

Example content

## 9.60  cs_vbit  Set video bitrate

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetVideoBitrate:(NSInteger)bitrate;

Set video bitrate
@param bitrate Video bitrate

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSInteger bitrate

[[XYBleManager sharedInstance] sendSetVideoBitrate:bitrate];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| bitrate | integer | Video bitrate, unit and range not specified |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_vbit |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.61  cs_blecfm  Play connected prompt

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendPlayBleConnectPromptTone;

Play Bluetooth connection prompt tone

### Usage

[[XYBleManager sharedInstance] sendPlayBleConnectPromptTone];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_blecfm","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_blecfm |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.62  cs_wbdc  Set BT disconnect on wear-off

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetWearBTDisconnect:(BOOL)isOn;

Set whether BT disconnects when glasses are taken off, default off
@param isOn Whether to open or not

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: BOOL isOn

[[XYBleManager sharedInstance] sendSetWearBTDisconnect:isOn];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| switch | integer | 1 on, 0 off |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_wbdc |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.63  cs_nwai  Allow AI while not worn

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetNotWearingAI:(BOOL)isOn;

Enable or disable AI while not wearing the glasses; command cs_nwai
@param isOn YES: enable (switch=1), NO: disable (switch=0)

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: BOOL isOn

[[XYBleManager sharedInstance] sendSetNotWearingAI:isOn];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| switch | integer | 1 on, 0 off |

B encoding: JSON object

{"C":"cs_nwai","B":{"switch":0}}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_nwai |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

{"C":"sr_nwai","S":0,"B":{"switch":0}}

## 9.64  cs_woft  Set wear-off delay

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetWearOffTime:(NSInteger)time;

Set the wear-off detection timer; command cs_woft
@param time Time in seconds, e.g. 5; valid range depends on device firmware

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSInteger time

[[XYBleManager sharedInstance] sendSetWearOffTime:time];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| time | integer | Seconds, legacy reference 1 to 120, SDK defers range to firmware |

B encoding: JSON object

{"C":"cs_woft","B":{"time":5}}

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_woft |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

{"C":"sr_woft","S":0,"B":{"time":5}}

## 9.65  cs_syrd  Read system readiness

Basis: SDK source.

- (void)sendGetSystemStatus;

Check whether the system is ready

### Usage

[[XYBleManager sharedInstance] sendGetSystemStatus];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_syrd","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.66  cs_flrv  Acknowledge received file

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendFileRecv:(XYFileType )fileType fileName:(NSString *)fileName;

Reports that a file has been received
@param fileType File type, XYFileTypeAll is unavailable
@param fileName File name

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: XYFileType fileType, NSString * fileName

[[XYBleManager sharedInstance] sendFileRecv:fileType fileName:fileName];

The SDK omits JSON string quotes around fname. File acknowledgment needs correction and verification. The SDK audio type is 51.

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| ftype | integer | 49 photo, 50 video, 51 audio, 52 BLE photo, never 255 |
| fname |  | Received file name, encoding issue in current implementation |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.67  cs_btna  Set BT name

Basis: Historical protocol 1.0.8; outbound parameters checked against source.

- (void)sendSetBTName:(NSString *)btName;

Set the BT Bluetooth name
@param btName BT Bluetooth name

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSString * btName

[[XYBleManager sharedInstance] sendSetBTName:btName];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| btname | string | New BT name |
| type | integer | Fixed at 0 by the SDK |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code | sr_btna |
| Completion criteria |  |

The reference protocol defines no business reply parameters.

Reply example

Example content

## 9.68  cs_svad  Set directional voice pickup

Basis: SDK source.

- (void)sendSetSvad:(BOOL )isOn;

Set to enable or disable directional sound pickup
@param isOn Whether to open or not

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: BOOL isOn

[[XYBleManager sharedInstance] sendSetSvad:isOn];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| enable | integer | 1 on, 0 off |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.69  cs_wfcon  Connect glasses to Wi-Fi

Basis: SDK source.

- (void)sendConnectWifi:(NSString *)ssid pwd:(NSString *)pwd;

Connect to the designated Wifi

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSString * ssid, NSString * pwd

[[XYBleManager sharedInstance] sendConnectWifi:ssid pwd:pwd];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| ssid | string | Wi-Fi SSID |
| pwd | string | Wi-Fi password |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.70  cs_syntm  Synchronize time string

Basis: SDK source.

- (void)sendSyncTime:(NSString *)time;

Synchronization time

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSString * time

[[XYBleManager sharedInstance] sendSyncTime:time];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| time | string | Time string, format not specified |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.71  cs_bleph  Take BLE photo

Basis: SDK source.

- (void)sendTakeBlePhoto:(NSString *)pictureName width:(NSInteger)width height:(NSInteger)height quality:(NSInteger)quality;

Ble photography
@param pictureName Picture name
@param width Picture width
@param height Picture height
@param quality Picture quality

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSString * pictureName, NSInteger width, NSInteger height, NSInteger quality

[[XYBleManager sharedInstance] sendTakeBlePhoto:pictureName width:width height:height quality:quality];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| fname | string | Photo name |
| width | integer | Width, supported range not specified |
| height | integer | Height, supported range not specified |
| quality | integer | Quality, range not specified |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.72  cs_en_campa  Enable custom camera parameters

Basis: SDK source.

- (void)sendEnCampa:(BOOL)isOn;

Set camera custom parameter mode
@param isOn Whether to open or not

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: BOOL isOn

[[XYBleManager sharedInstance] sendEnCampa:isOn];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| on | integer | 1 on, 0 off |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.73  cs_set_campa  Set custom camera parameters

Basis: SDK source.

- (void)sendCampaWithEV:(NSInteger)ev et:(long long)et iso:(NSInteger)iso awb:(NSInteger)awb;

Set camera parameters
@param ev EV parameter
@param et ET parameter
@param iso ISO parameter
@param awb AWB parameter

### Usage

Prepare these variables according to the request parameter descriptions; this is a call template: NSInteger ev, long long et, NSInteger iso, NSInteger awb

[[XYBleManager sharedInstance] sendCampaWithEV:ev et:et iso:iso awb:awb];

### Request parameters

| Field | Type | Description |
| --- | --- | --- |
| ev | integer | Exposure compensation parameter |
| et | integer | Exposure time parameter, unit not specified |
| iso | integer | ISO parameter |
| awb | integer | White balance parameter |

B encoding: JSON string; empty string when no data is present

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.74  cs_get_campa  Read camera parameters

Basis: SDK source.

- (void)sendGetCampa;

Get current camera parameters

### Usage

[[XYBleManager sharedInstance] sendGetCampa];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_get_campa","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## 9.75  cs_get_camparange  Read camera parameter ranges

Basis: SDK source.

- (void)sendGetCamparange;

Get camera parameter ranges

### Usage

[[XYBleManager sharedInstance] sendGetCamparange];

### Request parameters

No method payload parameters.

B encoding: JSON string; empty string when no data is present

{"C":"cs_get_camparange","B":""}

### Reply

| Item | Value |
| --- | --- |
| Reply code |  |
| Completion criteria |  |

| Field | Type | Description |
| --- | --- | --- |

Reply example

Example content

## Other public APIs and data models

Command methods appear in this chapter. The following completes the connection, callback, transfer, update and model APIs. Together these sections cover 94 public declarations.

## XYBleManager

### sharedInstance

+ (XYBleManager *)sharedInstance;

Singleton

### xy_sdkVersion

- (NSString *)xy_sdkVersion;

SDK version

### xy_startLog:

- (void)xy_startLog:(BOOL )isStart;

Start log
@param isStart Whether to enable the log

### xy_startScanBleDevice

- (void)xy_startScanBleDevice;

Start scanning device

### xy_stopScanBleDevice

- (void)xy_stopScanBleDevice;

Stop scanning device

### xy_connectBleDevice:

- (void)xy_connectBleDevice:(CBPeripheral *)peripheral;

Connecting device
@param peripheral peripheral

### xy_cancelConnectBleDevice:

- (void)xy_cancelConnectBleDevice:(CBPeripheral *)peripheral;

Unconnect device
@param peripheral peripheral

### xy_cancelAllConnect

- (void)xy_cancelAllConnect;

Cancel all connected devices

### xy_isConnectBleDevice

- (BOOL)xy_isConnectBleDevice;

Check whether peripherals are connected

### sendCustomDataHex:

- (void)sendCustomDataHex:(NSString *)dataHex;

Send a custom Hex instruction
@param dataHex Instruction Hex

### discoverDeviceBlock

@property (nonatomic, copy) void(^discoverDeviceBlock)(CBPeripheral *peripheral, NSDictionary *advertisementData, NSNumber *RSSI);

The callback of the device discovered by scanning

### connectStatusBlock

@property (nonatomic, copy) void(^connectStatusBlock)(BOOL isConnect);

Connection status callback

### receiveOriginalDataBlock

@property (nonatomic, copy) void(^receiveOriginalDataBlock)(NSData *respondData);

Receive original data callback

### receiveDataBlock

@property (nonatomic, copy) void(^receiveDataBlock)(XYBleRespondModel *respondModel);

Receive data callback

### lc3ReceiveDataBlock

@property (nonatomic, copy) void(^lc3ReceiveDataBlock)(NSData *lc3Data);

Receive LC3 audio data callback

### fileReceiveDataBlock

@property (nonatomic, copy) void(^fileReceiveDataBlock)(NSData *fileData);

File receive data callback

## XYSocketManager

### sharedInstance

+ (XYSocketManager *)sharedInstance;

Singleton

### xy_connectSocket

- (void)xy_connectSocket;

Connect

### connectStatusBlock

@property (nonatomic, copy) void(^connectStatusBlock)(NSError * _Nullable error);

Connection status callback

### receiveDataBlock

@property (nonatomic, copy) void(^receiveDataBlock)(BOOL isCompleted, XYFileDataType fileDataType, NSString *fileName, NSData *data, NSInteger fileSize);

Receive data callback

## XYOTAManager

### sharedInstance

+ (XYOTAManager *)sharedInstance;

Singleton

### xy_connectSocket

- (void)xy_connectSocket;

Connect

### xy_disconnectSocket

- (void)xy_disconnectSocket;

Disconnect

### xy_startOTAWithData:type:

- (void)xy_startOTAWithData:(NSData *)data type:(XYOTADataType)type;

Start OTA transfer

### connectStatusBlock

@property (nonatomic, copy) void(^connectStatusBlock)(NSError * _Nullable error);

Connection status callback

### sendProgressBlock

@property (nonatomic, copy) void(^sendProgressBlock)(NSInteger packIndex, NSInteger packNum, NSInteger currentSize, NSInteger fileSize);

OTA progress callback

### completionBlock

@property (nonatomic, copy) void(^completionBlock)(BOOL success, NSError * _Nullable error);

OTA completion callback

## XYBleRespondModel

### cmd

@property (nonatomic, copy) NSString *cmd;

Cmd

### status

@property (nonatomic, copy) NSString *status;

Status

### data

@property (nonatomic, strong) NSDictionary *data;

Data

### originalData

@property (nonatomic, strong) NSData *originalData;

Original Data

## XYFileRespondModel

### fileName

@property (nonatomic, copy) NSString *fileName;

File name

### fileType

@property (nonatomic, assign) NSInteger fileType;

File type：0：Photo，1：Video，2：Audio

### fileSize

@property (nonatomic, assign) NSInteger fileSize;

File size

### fileData

@property (nonatomic, strong) NSData *fileData;

File data

## XYDataUtil

### xy_convertStreamToFileDataWithData:

+ (XYFileRespondModel *)xy_convertStreamToFileDataWithData:(NSData *)data;

Convert stream data to file data
@param data Data

## Enumerations

Enum values and protocol values may differ; the SDK performs the conversion.

## XYFileType

typedef enum : NSUInteger {

XYFileTypePhoto, ///< Photo

XYFileTypeVideo, ///< Video

XYFileTypeAudio, ///< Audio

XYFileTypeBle, ///< BLE Photo

XYFileTypeAll, ///< All

} XYFileType;

## XYPhotoArgumentType

typedef enum : NSUInteger {

XYPhotoArgumentType1280_720, ///< 1280x720

XYPhotoArgumentType1920_1080, ///< 1920x1080

XYPhotoArgumentType2560_1920, ///< 2560x1920

XYPhotoArgumentType3840_2160, ///< 3840x2160

XYPhotoArgumentType4208_3120, ///< 4208x3120

} XYPhotoArgumentType;

## XYVideoArgumentSizeType

typedef enum : NSUInteger {

XYVideoArgumentSizeType1920_1080, ///< 1920x1080

XYVideoArgumentSizeType1280_720, ///< 1280x720

XYVideoArgumentSizeType640_480, ///< 640x480

} XYVideoArgumentSizeType;

## XYVideoArgumentQualityType

typedef enum : NSUInteger {

XYVideoArgumentQualityTypeLow, ///< Low-0

XYVideoArgumentQualityTypeMiddle, ///< Middle-1

XYVideoArgumentQualityTypeHigh, ///< High-2

} XYVideoArgumentQualityType;

## XYVideoArgumentDurationType

typedef enum : NSUInteger {

XYVideoArgumentDurationType30, ///< 30S

XYVideoArgumentDurationType60, ///< 60S

XYVideoArgumentDurationType180, ///< 180S

XYVideoArgumentDurationType300, ///< 300S

} XYVideoArgumentDurationType;

## XYAudioArgumentDurationType

typedef enum : NSUInteger {

XYAudioArgumentDurationType60, ///< 60S

XYAudioArgumentDurationType180, ///< 180S

XYAudioArgumentDurationType300, ///< 300S

XYAudioArgumentDurationType600, ///< 600S

XYAudioArgumentDurationType1800, ///< 1800S

XYAudioArgumentDurationType3600, ///< 3600S

XYAudioArgumentDurationType7200, ///< 7200S

} XYAudioArgumentDurationType;

## XYAIStatusType

typedef enum : NSUInteger {

XYAIStatusType_Start, ///< Start

XYAIStatusType_Chat, ///< In the conversation

XYAIStatusType_Stop, ///< Stop

XYAIStatusType_No_Sound, ///< Silence

} XYAIStatusType;

## XYFileDataType

typedef enum : NSUInteger {

XYFileDataTypePhoto, ///< Photo

XYFileDataTypeVideo, ///< Video

XYFileDataTypeAudio, ///< Audio

} XYFileDataType;

## XYOTADataType

typedef enum : NSUInteger {

XYOTADataTypeOTA_APK = 0,   ///< APK

XYOTADataTypeOTA_MTK = 1,   ///< MTK

XYOTADataTypeOTA_BES = 2,   ///< BES

} XYOTADataType;

# 10  Reply status and error handling

The following top-level S definitions come from the integration reference. Business state fields, socket NSError values and S belong to different layers and must be handled separately. Unknown event replies remain blank.

| Status | Meaning |
| --- | --- |
| 0 | Success |
| 1 | System not ready |
| 2 | Camera busy |
| 3 | Low battery |
| 4 | File missing |
| 5 | Photo capture failed |
| 6 | Storage full |
| 7 | Audio recording active |
| 8 | Video recording active |
| 9 | Wi-Fi connection failed |

# 11  Using the demo

Use XingYi_Demo from the matching delivery package. Run pod install in its root and open XingYi_Demo.xcworkspace. Configure the signing team, Bundle Identifier and hotspot capability, then build and run on a physical iPhone.

Validate module import, permissions, discovery, connection and battery queries first. Then test photos, file storage, reconnection, LC3 and OTA. UI libraries and third-party AI services are demo application features; configure them as needed.

Do not copy implementations that overwrite singleton callbacks across screens, use a fixed hotspot delay, equate nil error with connection success or equate transfer completion with installation success.

# 12  Updates and migration

This is the initial release of the developer documentation. There are no migration requirements from an earlier release. Begin with download and integration, followed by quick start.

Future releases will record feature and API changes, compatibility and migration steps in the developer center. Regenerate both manuals after updating commands and replies.